Experiment 2:

Objectives:

1.Throughput vs Number of IoE devices

2.Latency vs Number of IoE devices

3.Reliability vs Number of IoE devices

4.Energy Efficiency vs Number of IoE devices

Program:

import matplotlib.pyplot as plt

# Number of IoE devices
devices = [10, 50, 100, 200, 300, 400, 500]

# 5G network parameters (assumed)
capacity = 100                 # Mbps
base_latency = 10              # ms
packet_success_rate = 0.98
energy_per_packet = 0.001      # Joules

# IoE device parameters
packet_size = 1200             # Bytes
packets_per_second = 20
simulation_time = 10           # Seconds

# Store results
throughput = []
latency = []
reliability = []
energy_efficiency = []

for n in devices:

    # Total packets generated
    total_packets = n * packets_per_second * simulation_time

    # Offered traffic load (Mbps)
    offered_load = (
        n * packets_per_second * packet_size * 8 / 1e6
    )

    # Network utilization
    utilization = offered_load / capacity

    # 1. Throughput (Mbps)
    t = min(
        offered_load * packet_success_rate,
        capacity
    )

    # 2. Latency (ms): increases with congestion
    u = min(utilization, 0.95)
    l = base_latency + 10 * u / (1 - u)

    # 3. Reliability (%): decreases with congestion
    r = 100 * packet_success_rate * max(
        0, 1 - 0.2 * utilization
    )

    # 4. Energy efficiency (Mbit/J)
    successful_packets = total_packets * r / 100

    total_energy = (
        total_packets
        * energy_per_packet
        * (1 + utilization)
    )

    e = (
        successful_packets * packet_size * 8
        / total_energy / 1e6
    )

    # Store values
    throughput.append(t)
    latency.append(l)
    reliability.append(r)
    energy_efficiency.append(e)

    # Display results
    print(f"\nIoE Devices: {n}")
    print(f"Offered Load       : {offered_load:.2f} Mbps")
    print(f"Throughput         : {t:.2f} Mbps")
    print(f"Latency            : {l:.2f} ms")
    print(f"Reliability        : {r:.2f} %")
    print(f"Energy Efficiency  : {e:.2f} Mbit/J")

# Plot all four performance metrics
metrics = [
    (throughput, "Throughput (Mbps)"),
    (latency, "Latency (ms)"),
    (reliability, "Reliability (%)"),
    (energy_efficiency, "Energy Efficiency (Mbit/J)")
]

fig, axes = plt.subplots(2, 2, figsize=(11, 8))

for ax, (values, title) in zip(axes.flat, metrics):
    ax.plot(devices, values, marker="o")
    ax.set_title(title)
    ax.set_xlabel("Number of IoE Devices")
    ax.set_ylabel(title)
    ax.grid(True)

plt.suptitle("5G Performance Benchmarking for IoE Applications")
plt.tight_layout()
plt.show()

Output: