Experiment 2:

Simulation and Comparative
Performance Evaluation of Zigbee, 6LoWPAN and NB-IoT Communication Protocols
for IoE Applications    (Sample code)

Objectives:

1.Zigbee: BER vs Distance

2.6LoWPAN: BER vs Distance

3.NB-IoT Communication: BER vs Distance

Program:

import numpy as np
import matplotlib.pyplot as plt
from scipy.special import erfc

distance = np.linspace(10, 1000, 100)

# =====================================================
# Common BER equation
# BER = 0.5 * erfc(sqrt(SNR))
# =====================================================

# =====================================================
# 1. ZIGBEE
# =====================================================

Pt = 0          # Transmit power (dBm)
PL0 = 40        # Path loss at 1 m (dB)
n = 3.0         # Path loss exponent
N = -100        # Noise power (dBm)

PL = PL0 + 10*n*np.log10(distance)
Pr = Pt - PL
SNR = Pr - N

BER = 0.5 * erfc(np.sqrt(10**(SNR/10)))

plt.figure(figsize=(7,5))
plt.semilogy(distance, BER)
plt.xlabel("Distance (m)")
plt.ylabel("Bit Error Rate (BER)")
plt.title("Zigbee: BER vs Distance")
plt.grid(True)
plt.show()


# =====================================================
# 2. 6LoWPAN
# =====================================================

Pt = 0
PL0 = 40
n = 3.0
N = -100

PL = PL0 + 10*n*np.log10(distance)
Pr = Pt - PL
SNR = Pr - N

BER = 0.5 * erfc(np.sqrt(10**(SNR/10)))

plt.figure(figsize=(7,5))
plt.semilogy(distance, BER)
plt.xlabel("Distance (m)")
plt.ylabel("Bit Error Rate (BER)")
plt.title("6LoWPAN: BER vs Distance")
plt.grid(True)
plt.show()


# =====================================================
# 3. NB-IoT
# =====================================================

Pt = 23
PL0 = 31
n = 3.5
N = -120

PL = PL0 + 10*n*np.log10(distance)
Pr = Pt - PL
SNR = Pr - N

BER = 0.5 * erfc(np.sqrt(10**(SNR/10)))

plt.figure(figsize=(7,5))
plt.semilogy(distance, BER)
plt.xlabel("Distance (m)")
plt.ylabel("Bit Error Rate (BER)")
plt.title("NB-IoT: BER vs Distance")
plt.grid(True)
plt.show()

Output: